LINKEDIN VIDEO SCRIPT

Lithium-ion 18650 Cell — Structural Breakdown & Layer Demonstration

⏱ 0:00 – 0:03 — Title Frame

ON-SCREEN TEXT (center):
“Lithium-ion 18650 Cell”

⏱ 0:04 – 0:10 — 360° Cell Showcase

ON-SCREEN LABELS:

- “18650 Li-ion Cell”

- “Cylindrical Steel Can”

- “Positive Terminal (Top Cap)”

- “Negative Terminal (Can Body)”

(add arrows pointing to top cap & bottom body)

⏱ 0:11 – 0:18 — Multimeter Continuity Test

ON-SCREEN TEXT:

- “Checking Terminal Connections”

- “Outer Steel Can = Negative Terminal”

- “Top Cap = Positive Terminal”

- “Continuity detected at both ends”

(Show continuity symbol / “Beep” icon)

⏱ 0:19 – 0:26 — Cutting the Positive Side

ON-SCREEN SAFETY LABEL:
⚠️ “Performed under controlled conditions — Do NOT try this at home.”

ON-SCREEN TECH LABELS:

- “Internal Insulation Ring”

- “Positive Cap Spot Welds”

(arrows pointing to the plastic membrane & welds)

⏱ 0:27 – 0:34 — First View of Internal Jelly-Roll

ON-SCREEN LABELS:

- “Jelly-Roll Structure ”

⏱ 0:35 – 0:41 — Showing Negative Terminal Side

ON-SCREEN TEXT:

- “Negative Terminal Connection”

- “Anode Copper Tab → Welded to Steel Can”

(add arrow to bottom weld)

⏱ 0:42 – 0:50 — Cutting the Bottom Side

ON-SCREEN LABELS:

- “Bottom Weld Connection Visible”

- “Copper Tab (Anode)”

⏱ 0:51 – 0:58 — Quick Layer Show (1-second per layer)

ON-SCREEN LABELS:
(each displayed for 1 sec)

- “Anode Sheet – Graphite on Copper Foil”

- “Separator – Microporous Polymer Film (PE/PP)”

- “Cathode Sheet – Lithium Metal Oxide on Aluminum”

- “Separator – Microporous Polymer Film (PE/PP)”

⏱ 0:59 – 1:10 — Full Unroll of the Jelly-Roll

ON-SCREEN TEXT:

- “Complete Unrolled Jelly-Roll”

- “Anode + Separator + Cathode in Spiral Winding”

(Maybe use a top-bar text: “Typical Length: ~1 meter depending on capacity”)

⏱ 1:11 – 1:20 — Structural Summary

ON-SCREEN BULLET BOX / TEXT PANEL:

“18650 Cell Internal Components”

- Steel Can → Negative Terminal

- Positive Cap → Isolated via Insulation Ring

- Cathode (Aluminum foil coated)

- Anode (Copper foil coated)

- Separator Film

- Electrolyte Soaked Layers

- Welded Tabs (Positive & Negative)

⏱ 1:21 – 1:25 — Closing Frame + Disclaimer

ON-SCREEN TEXT:
“Li-ion Cell Teardown for Educational Purpose Only”
“Handled in Controlled Environment”